AIR BOSS 102

Recurrency Training

STUDENT WORKBOOK

| Participant Name |  |
| --- | --- |
| LOA Level | BAB · SAB · RAB/SV · RAB/MV (circle one) |
| Date |  |
| Location |  |
| Instructor |  |
| Revision | 1.5 — July 2026 |

This workbook is your personal learning record for Air Boss 102. Complete each section during the session. Keep it as a reference after training.

How to Use This Workbook	3

Module 1 — Accident & Incident Review (A)	4

Module 2 — FAA Focus Items (B)	7

Module 3 — Program Changes: ABRP, ACE, FAST, FAST-Signatory, SMOs (C)	10

Module 4 — FAA Order 8900.1 Changes (D)	12

Module 5 — Emergency Response Readiness (E)	14

Module 6 — Airport & Ground Operations Integration (F)	17

Module 7 — Military Team Participants (G)	19

Module 8 — CRM and Human Factors (H)	21

Module 9 — Instruction & Evaluation (I)	24

Module 10 — Standardized Air Boss Phraseology / Lexicon (J)	27

Appendix A — Glossary	30

Appendix B — Standardized Air Boss Lexicon	34

Additional Notes — Page 1	38

Additional Notes — Page 2	40

Additional Notes — Page 3	42

How to Use This Workbook

This workbook accompanies the Air Boss 102 instructor-led session. It is organized to follow the module sequence in the course deck. Use it to:

- Capture key facts, personal notes, and action items in each module.

- Complete the fillable worksheets and checklists during instruction.

- Work through discussion questions and written exercises in Modules 1, 4, and 7.

- Reference the Glossary (Appendix A) and Standardized Lexicon (Appendix B) before and after the course.

Icons used throughout:

| ✏ WRITE | Space for your notes or written responses. |
| --- | --- |
| □ CHECK | Checklist item — mark when confirmed or completed. |
| • KEY POINT | Numbered teaching point from the deck — fill in the blank or confirm. |

Passing score for the written assessment: 80% cumulative on all MCQ components (ABRP Manual § 11). The exam follows Module 10. Reference materials are closed during the exam.

Module 1 — Accident & Incident Review (A)

MODULE 1 (A) Accident & Incident Review 45 minutes · Case studies · Lessons Applied

Use the 5M lens (Man / Machine / Medium / Mission / Management) to analyze the case studies below before completing the causal-factor table in 1.2. The 5M model and the four causal-factor categories are two different tools — don't conflate them.

Case Study Analysis Framework — The 5M Model

| Dimension | Notes — apply while reviewing each case study below |
| --- | --- |
| MAN |  |
| MACHINE |  |
| MEDIUM |  |
| MISSION |  |
| MANAGEMENT |  |

1.1 Case Study Notes

Laredo, TX — A-20G Havoc (N747HS), Feb 16 2025  |  NTSB Case CEN25LA097

Findings are preliminary. The NTSB final report is pending.

| Causal factors identified (fill in) | Air Boss positive action (fill in) |
| --- | --- |

Notes on the Laredo sequence:

Wings Over Dallas (2022) — NTSB Final Report AIR-24-07 (Dec 9, 2024)

NTSB Safety Recommendation A-24-35: Asks ICAS, with the FAA and industry, to develop standardized, unambiguous air boss directives. Basis for Module 10 and the ICAS Lexicon (discussion draft).

Probable cause (final): Inadequate prebriefed aircraft separation plan; reliance on real-time air boss deconfliction and see-and-avoid — which failed as the B-17G and P-63F flight paths converged. Key lesson for air boss communications:

International Case Study (current cycle — filled in by instructor)

| Incident: |  |
| --- | --- |

| Date / Location: |  |
| --- | --- |

Causal factors and lessons:

1.2 Causal Factor Framework

Complete the table with one air boss mitigation for each causal factor category:

| Causal Factor | Air Boss Mitigation |
| --- | --- |
| Pilot Error |  |
| Communication Gap |  |
| Airspace Incursion |  |
| Ground Ops Conflict |  |

1.3 Lessons Applied — Replay & Rewrite

Work in pairs. Apply the causal factor framework to the assigned incident. Then identify which lexicon terms would have applied.

Step 1 — Identify the earliest intervention point

Your answer:

Step 2 — Re-write the Air Boss radio call using standardized terms

Use HOLD, KNOCK IT OFF, and/or CLEARED IN as appropriate. Follow word order: callsign · directive · reference · value.

Your revised call:

Step 3 — Group debrief notes

Module 2 — FAA Focus Items (B)

MODULE 2 (B) FAA Focus Items 45 minutes

2.1 Current FAA National Emphasis Items

Complete during instruction. Update before each training cycle from current AFS-830 guidance.

1.

2.

3.

4.

2.2 Common FAA Surveillance Deficiency Categories

From the Module 2 compliance checklist, record the three most common deficiencies:

| # | Deficiency — write in from the deck |
| --- | --- |
| 1 |  |
| 2 |  |
| 3 |  |

2.3 Pre-Show Compliance Checklist

Mark each item when confirmed. Bring this checklist to every show.

Communications

□  Primary radio confirmed operable before the first act

□  Backup radio staged at the air boss stand and tested

□  Guard frequency (121.5) monitoring assigned

□  Radio check with all performers completed

□  Show frequencies (primary and backup) posted at the air boss stand

Airspace & Waiver

□  Certificate of Waiver / Authorization (CoW/A) on site and reviewed

□  Performer profiles verified against CoW/A categories

Emergency Response

□  IC or Unified Command member physically co-located at air boss stand

□  ARFF/CFR 1-minute deployment standard confirmed and staged

□  ERP distributed to all stakeholders; emergency frequencies confirmed

□  Aerobatic box access routes clear and marked for ARFF

Performer Credentials

□  All performer SAC cards reviewed and valid

□  FAST/FAST-Signatory documentation confirmed for formation acts

□  International performer credentials (Transport Canada / CASA) verified if applicable

□  Military team profile and run-in geometry briefed and approved

Ground & Ramp

□  GOO on dedicated frequency and briefed on ramp access zones

□  Ramp access zones (red / amber / green) posted and enforced

□  Media/photography crew briefing completed and zones marked

□  Fueling operations away from active areas confirmed with FBO

2.4 Personal Notes

Module 3 — Program Changes: ABRP, ACE, FAST, FAST-Signatory, SMOs (C)

MODULE 3 (C) Program Changes 45 minutes

3.1 My LOA Level and Scope

| My current LOA level BAB / SAB / RAB/SV / RAB/MV (circle) | LOA expiry date |
| --- | --- |

One operational constraint specific to my LOA level:

3.2 ABRP Manual v2.0 (Dec 2025) — Key Changes

Record updates from the deck that are relevant to your LOA level.

| Section / Topic | What changed / what it means for me |
| --- | --- |
| § 1 — Definitions |  |
| § 8 — ABI Standards |  |
| § 9 — ABE Standards |  |
| § 11 — Recurrency Requirements |  |
| LOA Pathways |  |

3.3 FAST / FAST-Signatory Verification Checklist

Use this checklist before sanctioning any formation act that requires FAST coordination:

□  Team identified as requiring FAST coordination (confirm with deck or current ICAS guidance)

□  FAST-Signatory name and current designation confirmed in writing before the briefing

□  Signatory documentation current — expiration date verified:

□  Team formation standards briefed and any deviations from standard approved by Signatory

□  Formation act profile reviewed against CoW/A permitted categories

□  Lost-comm and abort criteria briefed to all formation members

3.4 Notes

Module 4 — FAA Order 8900.1 Changes (D)

MODULE 4 (D) FAA Order 8900.1 Changes 45 minutes · Mini-Exercise included

4.1 Order 8900.1 — Key Sections for Air Bosses

Current change reference: CHG 694 (verify each cycle). Vol. 5 Ch. 9. No significant changes confirmed this cycle.

| Section | Key requirement / what the air boss must confirm |
| --- | --- |
| § 3-154A | Event organizer / military profile review |
| § 3-154B | ERP requirements — ARFF 1-min standard; IC co-location during entire CoW/A |
| § 3-154C | Required safety briefing elements (Communications, Emergency Procedures, Weather/Waiver minimums) |

From the list your instructor presents, mark each item that IS required by 8900.1 § 3-154C. Circle your selection, then confirm against the answer key in the debrief.

Required items (circle from the list above, then confirm in debrief):

4.2 UAS/COA Deconfliction — Discussion Question

✏  WRITE: A media drone operating under a COA loses link during a low-level jet pass. Its return-to-home path crosses the aerobatic box. Write your first three air boss transmissions and actions:

| Transmission 1: |  |
| --- | --- |

| Transmission 2: |  |
| --- | --- |

| Transmission 3: |  |
| --- | --- |

Additional actions (IC notification, ATC coordination, documentation):

Module 5 — Emergency Response Readiness (E)

MODULE 5 (E) Emergency Response Readiness 45 minutes · ERP Standards

5.1 ERP Validation Checklist — Practice/Rehearsal Day

Use this checklist on the practice day to confirm ERP readiness before the CoW/A opens:

□  IC or Unified Command member physically co-located at the air boss stand

□  ARFF/CFR confirmed staged; 1-minute standard verified for the announced window (aircraft flying) — confirm the 3-minute standard applies instead for an unannounced window with the CoW/A in effect and no aircraft flying

□  EMS staged and confirmed on show frequency / dedicated emergency frequency

□  Emergency access routes confirmed clear and marked

□  IC notified and confirmed of aerobatic box dimensions and show sequence timing

□  Pyro safety officer (if applicable) confirmed and on assigned frequency

□  Parachute landing zone (LZ) confirmed clear of obstructions and obstacles

□  All-clear codes confirmed with all emergency stakeholders

□  ERP tabletop or full-scale exercise completed; after-action report initiated

5.2 High-Risk Scenario Response — Four Templates

| Scenario | Your first three radio transmissions / actions |
| --- | --- |
| Midair Collision |  |
| Aircraft Down / Crash |  |
| Spectator / Ground Injury |  |
| Pyro Misfire / Fire |  |

5.3 After-Action Report Template

Complete after any ERP activation or exercise. Retain for IIC and ICAS program manager records.

| Date / Show: |  |
| --- | --- |

| Scenario type: |  |
| --- | --- |

| Time of initial air boss transmission: |  |
| --- | --- |

| Time IC was notified: |  |
| --- | --- |

| Time ARFF arrived in the aerobatic box: |  |
| --- | --- |

Summary of actions taken:

Gaps or deficiencies identified:

Corrective actions before the next show day:

Module 6 — Airport & Ground Operations Integration (F)

MODULE 6 (F) Airport & Ground Operations Integration 45 minutes

6.1 Stakeholder Integration Matrix

Fill in the primary coordination task and the show-day communication channel for each stakeholder at your show.

| Stakeholder | Primary pre-event coordination task | Show-day comms channel |
| --- | --- | --- |
| Airport Manager |  |  |
| FBO |  |  |
| ATC Facility |  |  |
| Security / Law Enforcement |  |  |
| Event Organizer |  |  |

6.2 Ramp Access Zone Protocol

Record the access zone definitions and the check-in gate protocol for your show:

| Zone | Definition / Access rules during CoW/A |
| --- | --- |
| RED | No unauthorized access during CoW/A |
| AMBER | Restricted access — escort required; sign in with GOO |
| GREEN | General event access |

6.3 Ground Ops Personal Notes

6.3  Specialty Ground & Airspace Elements

Mark each item you have confirmed for your show:

□  UAS: GCS and launch/recovery sited away from crowd and ground-ops flow; altitude deconfliction with manned acts briefed

□  Pyrotechnics: shot lines and fallout buffers confirmed against crowd line and ground personnel; SIC go/no-go channel confirmed with Air Boss

□  Car races / jet cars: runway or taxiway closure and crossing timing coordinated with Air Boss; ground vehicle speed and course boundaries briefed same as aircraft acts

Module 7 — Military Team Participants (G)

MODULE 7 (G) Military Team Participants 45 minutes

7.1 Military Team Reference Card

Complete during instruction. Update each cycle from current show advisories.

Resource: team advisories, run-in geometry, and host requirements are published annually on each team’s FAA/DoD site. Download the current-year documents and use AI tools to help build your coordination timeline.

| Team | Aircraft | Special coordination requirement | Altitude format |
| --- | --- | --- | --- |
| USN Blue Angels | F/A-18E/F Super Hornet (×6 jets) + C-130T Fat Albert | Winter Visit — BA #7/#8 on-site, Jan–Mar; run-in approval each pass | Feet (imperial) |
| USAF Thunderbirds | F-16C/D Fighting Falcon (×8 jets) | Dedicated Project Officer POC (60 days prior); TFR MUST center on show center, not airfield center — wrong centering is a potential show-stopper; run-in approval each pass | Feet (imperial) |
| CAF Snowbirds | CT-114 Tutor — FINAL CT-114 SEASON 2026 (standing down after 11 Oct 2026) | Entry/exit COA; Transport Canada + 1 CAD brief | VERIFY metric vs. imperial before brief |
| Tactical Demo Teams (F-22, F-35, etc.) | Varies by cycle | Approved envelope review; profile brief before each act | Confirm per team advisory |

7.2 Run-In Line Approval — Discussion Question

✏  WRITE: The Blue Angels lead pilot announces a modified run-in due to sun angle. What do you do?

Step 1 — Is the modification within the briefed envelope? (Yes / No / Need to confirm)

Step 2 — If yes: action taken:

Step 3 — If no: transmission made:

7.3 DD Form 2535 Key Fields

From the deck walk-through, record the DD Form 2535 fields that directly affect air boss planning:

| Team identity and aircraft type: |  |
| --- | --- |

| Approved profile / envelope: |  |
| --- | --- |

| Emergency coordination point of contact: |  |
| --- | --- |

| Lost-comm visual signal sequence: |  |
| --- | --- |

Personal notes:

Module 8 — CRM and Human Factors (H)

MODULE 8 (H) CRM and Human Factors 45 minutes · Standard Word Order

8.1 Key Human Factors Concepts

Match each concept to its air boss application (fill in from the deck):

| Concept | Air boss application / mitigation |
| --- | --- |
| Error Chain Recognition |  |
| Plan Continuation Bias |  |
| Task Shedding / Delegation |  |
| Workload Saturation |  |
| Fatigue Management (multi-day shows) |  |

8.2 Standard Word Order

Scenario: Cobra is at show center, 2,000 ft, right-to-left. Mustang is inbound from the west at 1,500 ft. Write each air boss call. Follow word order: callsign · directive · reference · value.

| Step 1 — HOLD call to Mustang: |  |
| --- | --- |

| Expected Mustang readback: |  |
| --- | --- |

| Step 2 — TRAFFIC advisory to Cobra: |  |
| --- | --- |

| Expected Cobra readback: |  |
| --- | --- |

| Step 3 — Clear Mustang into the box (after Cobra clears): |  |
| --- | --- |

| Expected Mustang readback: |  |
| --- | --- |

What is the most common error in word order you made? How will you correct it?

8.3 Swiss Cheese Model — Laredo Mapping

Map the five layers of the Swiss Cheese model onto the Laredo accident (Module 1).

| Layer | What failed / was missing at Laredo |
| --- | --- |
| Organizational Influences |  |
| Unsafe Supervision |  |
| Preconditions for Unsafe Acts |  |
| Unsafe Acts |  |
| Defenses / Barriers |  |

8.4  Air Boss Substitution — Three Scenarios

Record the key requirement for each scenario (from slide 33):

Planned Handoff (two qualified LOA holders):

Best practice at complex shows — not an ICAS/FAA mandate: list both on CoW/A as Primary Air Boss. Pre-brief active acts, frequencies, IIC/IC contact, and weather minimums before the transfer. Show Day Credit: primary = 1 credit; split at handoff = 0.5 credit each.

Emergency Substitution (unplanned mid-event change):

Requirement: ______________________________________________________

Safety Advisor — ABRP § 10 (exigent circumstances):

Constraints: max 5 Aerial Acts; 1 act in waivered airspace at a time; surface SAC required; may not perform that Show Day; must consult ABI/ABE beforehand; requires FAA deviation through IIC.

Module 9 — Instruction & Evaluation (I)

MODULE 9 (I) Instruction & Evaluation 45 minutes

9.1 Daily Participants Safety Briefing — Required Elements Checklist

Mark each item as you cover it during a safety briefing. Use this at your own shows.

Administration Section

□  IIC badge number confirmed and recorded on the briefing record

□  Time hack given at briefing start; acknowledged by all performers

□  Performer sign-in completed; roster confirmed before first act

Weather Section

□  Current METAR reviewed; weather minimums stated

□  Go/no-go criteria confirmed with IIC and event organizer

Airspace Section

□  TFR effective times and lateral/vertical limits confirmed with ATC

□  Active runway status and potential conflicts stated to all performers

□  NOTAM published and confirmed current

Waiver / CoW/A Section

□  CoW/A on hand; provisions reviewed with the IIC

□  Performer SAC categories verified against CoW/A category limits

□  UAS/COA integration confirmed (if UAS operations planned)

Show Layout Section

□  Show center, corner markers, and crowd line reviewed with all performers

□  Aerobatic box boundaries confirmed; safety hotspots briefed

□  Show sequence and timing reviewed

Emergency Procedures Section

□  ARFF response posture and location stated

□  ARFF 1-minute deployment standard confirmed

□  KIO (KNOCK IT OFF) signal and phraseology stated for all performers

□  Airfield status — active runways, potential conflicts stated

Communications Section

□  Primary show frequency stated

□  Backup show frequency stated

□  Guard (121.5) monitoring responsibility assigned

□  Lost-comm procedures stated for each category of performer

□  Radio check with all performers completed before the first act

Time Hack & Other

□  Wake turbulence advisory given (if applicable)

□  Act-by-act sequence reviewed

□  Q&A completed before first departure

Lexicon confirmation (connects to Module 10):

□  Lexicon term set introduced and distributed (briefing card / kneeboard)

□  KNOCK IT OFF procedure confirmed — spoken three times, acknowledged by callsign

□  Left/right direction convention confirmed — crowd perspective

□  Mandatory readback requirement stated for TRAFFIC, HOLD, CONFLICT, KNOCK IT OFF

9.2 Plan of Action — Personal Draft

Use this template to draft your Plan of Action. The ABI reviews this before the show; the ABE reviews it during the evaluative conference.

| Show / Venue: |  |
| --- | --- |

| CoW/A Number: |  |
| --- | --- |

| Show Date(s): |  |
| --- | --- |

| My LOA Level: |  |
| --- | --- |

Emergency Response Plan reference (ERP document title, version, date):

Performer profiles (list each act, SAC category, and any special coordination notes):

Required briefing elements confirmed (list three from § 3-154C not already checked above):

Lexicon briefing confirmation — how will you introduce the lexicon at this show?

9.3 Evaluative Conference Prep — Key Questions

From the discussion, write one question you would want to be prepared to answer in an evaluative conference:

Module 10 — Standardized Air Boss Phraseology / Lexicon (J)

MODULE 10 (J) Standardized Air Boss Phraseology / Lexicon 30 minutes · Radio Practice

10.1 Six Usage Conventions

Fill in each convention from the deck:

| # | Convention — write in the rule as stated |
| --- | --- |
| 1 |  |
| 2 |  |
| 3 |  |
| 4 |  |
| 5 |  |
| 6 |  |

10.2 Radio Practice — Five Worked Examples

Callsigns used: Boss (air boss); Cobra, Mustang, Viper (performers). Write your call, then check it against the conventions above.

Example 1 — Launch (line up and wait, then cleared for takeoff)

| Your call (line up and wait): |  |
| --- | --- |

| Performer readback: |  |
| --- | --- |

| Your call (cleared for takeoff): |  |
| --- | --- |

Example 2 — Clearing into the box (HOLD HIGH → CLEARED IN)

| HOLD HIGH call: |  |
| --- | --- |

| Performer readback: |  |
| --- | --- |

| CLEARED IN call (state direction): |  |
| --- | --- |

Example 3 — Traffic advisory and acknowledgment

| TRAFFIC call (by callsign and position): |  |
| --- | --- |

| Required performer response (VISUAL or PADLOCKED): |  |
| --- | --- |

Example 4 — Emergency stop (KNOCK IT OFF)

Write the call exactly as specified by the lexicon:

How many aircraft callsigns must you hear before clearing the airspace?

Example 5 — Recovery (RECOVER and EXTEND)

| RECOVER call: |  |
| --- | --- |

| EXTEND call (if spacing needed): |  |
| --- | --- |

| CLEARED TO LAND call: |  |
| --- | --- |

10.3 Adoption Status — Record the key caveat

This lexicon is (circle): An adopted FAA/ICAS standard  |  A proposed working draft pending consensus adoption

NTSB recommendation driving Module 10:

Founding accident:

Appendix A — Glossary

Terms are drawn from ABRP Manual § 1 (v1.0 definitions; v2.0 expands these) and supplemented with terms used throughout this course. The authoritative source for each term is noted in the left column. When a term appears in both the ABRP Manual and FAA Order 8900.1, the ABRP Manual definition governs for ABRP program purposes.

ABRP Manual program authority: airshows.aero/GetDoc/4140  ·  ABRP Manual v2.0 (Dec 2025) supersedes all prior versions.

| Air Boss ABRP Manual § 1 | The individual who has the primary responsibility for air show operations on the active taxiways, runways, and the surrounding air show demonstration area (FAA 8900.1). ICAS adds: must prepare and present a quality comprehensive air show safety and sequence briefing, demonstrate mastery of FAA Form 7711-2, and have command and control presence. |
| --- | --- |

| Air Boss Evaluator (ABE) ABRP Manual § 1 | An air boss who holds a Letter of Authorization (LOA) and has been selected by ICAS to evaluate applicants in the ABRP. |
| --- | --- |

| Air Boss Instructor (ABI) ABRP Manual / Course | An individual approved under ABRP Manual § 8 to provide instruction to air boss candidates. ABIs conduct training, review Plans of Action, and support the evaluative conference process. |
| --- | --- |

| Air Boss Recommendation Committee (ABRC) ABRP Manual § 1 | A five-member sub-committee of the ICAS Safety and Operations Committee (SOC) that oversees and manages the ABRP. |
| --- | --- |

| Air Boss Recommendation Program (ABRP) ABRP Manual § 1 | An ICAS program that qualifies prospective air bosses to receive an ICAS recommendation to the FAA for a Letter of Authorization (LOA) to conduct air boss duties at public events in the United States. |
| --- | --- |

| Air Show ABRP Manual § 1 | An aviation event defined as an aerial demonstration by one or more aircraft before an invited assembly of persons (FAA 8900.1 definition adopted by ABRP). |
| --- | --- |

| BAB (Basic Air Boss) ABRP Manual / Course | The entry-level LOA in the ABRP tier structure. Authorized to conduct air boss duties at basic-category events as defined by the ABRC. See ABRP Manual v2.0 for current scope. |
| --- | --- |

| Certificate of Waiver / Authorization (CoW/A) FAA Order 8900.1 / AC 91-45C | FAA Form 7711-2. The waiver that defines the aerobatic box, show line, crowd separation, performer categories, and operating conditions. The air boss operates within the authority delegated by the CoW/A. |
| --- | --- |

| Certificate of Authorization (COA) FAA Order 8900.1 / AC 91-45C | An FAA authorization used for UAS operations and military entry/exit corridors. During the CoW/A window, COA holders must coordinate with the air boss before operating within the TFR. |
| --- | --- |

| Exception ABRP Manual § 1 | Permission to do or not to do something that others are obliged to do. Each requirement of the ABRP is subject to a request for exception to the ABRC. |
| --- | --- |

| FAST (Formation and Aerobatic Standardization Team) ABRP Manual / Course | The ICAS program that sanctions multi-ship formation aerobatic acts. Certain formation acts require FAST coordination. The air boss must confirm FAST-Signatory documentation before sanctioning the act. |
| --- | --- |

| FAST-Signatory ABRP Manual / Course | The specific individual who has signed off on a formation team's standards under the FAST program. Current signatory documentation must be confirmed at or before the performer safety briefing. |
| --- | --- |

| GOO (Ground Operations Officer) ABRP Manual / Course | The individual designated to manage ramp and ground safety during the show. Operates on a dedicated frequency under the air boss's authority. |
| --- | --- |

| IC (Incident Commander) FAA Order 8900.1 / AC 91-45C | The individual who heads the Incident Command System (ICS) at the show. Must be physically co-located with the air boss during the entire CoW/A window per FAA Order 8900.1 § 3-154B. |
| --- | --- |

| IIC (Inspector in Charge) FAA Order 8900.1 / AC 91-45C | The FAA inspector assigned as the on-site regulatory authority during the show. The IIC does not replace the air boss's delegated authority within the CoW/A but is the regulatory point of contact. |
| --- | --- |

| KIO / Knock It Off ABRP Manual / Course | The standard emergency-stop directive. In the standardized lexicon, KNOCK IT OFF is spoken three times and every affected aircraft must acknowledge by callsign before the air boss clears the airspace. |
| --- | --- |

| Letter of Authorization (LOA) ABRP Manual § 1 | A document issued by the FAA accrediting an individual to conduct air boss duties. There are four LOA levels in the ABRP tier structure: BAB, SAB, RAB/SV, and RAB/MV. See ABRP Manual v2.0 for current scope. |
| --- | --- |

| Mini Boss ABRP Manual § 1 | An individual who has demonstrated basic knowledge of the airport/aviation/air show environment, but has not yet demonstrated the experience level, qualifications, or demeanor to manage the briefing and air show flying operations. |
| --- | --- |

| RAB (Recognized Air Boss) ABRP Manual § 1 | An air boss who has been recommended by ICAS in accordance with the ABRP and has received a Letter of Authorization from the FAA. In the ABRP tier structure, RAB is divided into RAB/SV (Single Venue) and RAB/MV (Multi-Venue). |
| --- | --- |

| RAB/MV (Recognized Air Boss — Multi-Venue) ABRP Manual / Course | An LOA level authorizing the holder to serve as air boss at multiple show venues. Highest tier in the ABRP structure. See ABRP Manual v2.0 for current scope and prerequisites. |
| --- | --- |

| RAB/SV (Recognized Air Boss — Single Venue) ABRP Manual / Course | An LOA level restricting the holder to a specific, single show venue. Equivalent to the historical 'single site LOA.' See ABRP Manual v2.0 for current scope. |
| --- | --- |

| SAB (Single Aircraft Air Boss) ABRP Manual / Course | A mid-tier LOA level in the ABRP tier structure, authorizing the holder to conduct air boss duties for single-aircraft events as defined by the ABRC. See ABRP Manual v2.0 for current scope. |
| --- | --- |

| SAC (Statement of Aerobatic Competency) ABRP Manual / Course | The ACE Program credential held by each performer. The air boss verifies SAC card validity at the performer safety briefing before sanctioning any aerobatic act. |
| --- | --- |

| Safety and Operations Committee (SOC) ABRP Manual § 1 | A standing committee of ICAS that oversees the association's safety and operations functions, including the ABRP through the ABRC sub-committee. |
| --- | --- |

| Shadowing ABRP Manual § 1 | When a prospective air boss observes a recognized air boss for the purpose of learning how to become an air boss himself/herself. |
| --- | --- |

| Show Day ABRP Manual § 1 | A single calendar day in which an individual served as the principal air boss — i.e., briefing and controlling air show aircraft — at a specific venue. A typical venue with Friday, Saturday, and Sunday shows counts as three show days. |
| --- | --- |

| Single Site Letter of Authorization ABRP Manual § 1 | A letter of authorization that applies to a specific, single show site or venue. Equivalent to RAB/SV in the current ABRP tier structure. |
| --- | --- |

| SMO (Safety Management Organization) ABRP Manual / Course | Third-party organizations recognized by ICAS and/or Transport Canada to administer safety standards for certain categories of performers or events. Includes international bodies such as Transport Canada and CASA. |
| --- | --- |

| TFR (Temporary Flight Restriction) FAA Order 8900.1 / AC 91-45C | A FAA-issued airspace restriction that defines the lateral and vertical boundaries of the air show airspace. The air boss operates within the TFR as delegated by the CoW/A. |
| --- | --- |

| Venue ABRP Manual § 1 | A waivered air show site. The same location in a different year or with a different orientation or setup of the aerobatic box is NOT a different venue. |
| --- | --- |

Appendix B — Standardized Air Boss Lexicon

This reference set supports Module 10. Use it at the performer safety briefing and at the air boss stand.

ADOPTION STATUS CAVEAT This is a proposed working lexicon developed by ICAS in response to NTSB Safety Recommendation A-24-35, following the 2022 Wings Over Dallas mid-air collision. It is NOT an adopted FAA or ICAS standard. Any operational use must be reconciled with the specific CoW/A and performer briefing at each show. The NTSB recommendation calls for a consensus standard developed jointly by the FAA, ICAS, and industry; that process is ongoing.

Show-Line and Boundary Terms

| SHOW LINE Show-Line and Boundary Terms | The designated line parallel to the crowd that defines the nearest boundary of all aerobatic maneuvers. Usage: Never substitute 'the line.' |
| --- | --- |

| DEADLINE Show-Line and Boundary Terms | A hard boundary within the aerobatic box that no maneuver may cross. Used when a tighter limit than the show line is required. Usage: State which boundary: 'Cobra, DEADLINE, north end.' |
| --- | --- |

| SHOW CENTER Show-Line and Boundary Terms | The reference point at the midpoint of the show line used for positional calls. Usage: Use as the reference reference in all positional calls. |
| --- | --- |

| CROWD LINE Show-Line and Boundary Terms | The physical boundary separating spectators from the airside area. All performer clearances assume the crowd line as the baseline spectator position. Usage: Briefed at the safety briefing; confirmed on site. |
| --- | --- |

Positional and Directional Terms

| LEFT-TO-RIGHT / RIGHT-TO-LEFT Positional and Directional Terms | Direction of travel along the show line from the spectators' perspective. Left is to the left as a spectator faces the airshow area. Usage: Always crowd-perspective. Never pilot-perspective. |
| --- | --- |

| HOLD HIGH Positional and Directional Terms | Instruction to maintain altitude above a stated reference before entering the box. Always followed by a reference and value. Usage: Word order: '[Callsign], HOLD HIGH, [reference], [altitude].' |
| --- | --- |

| EXTEND Positional and Directional Terms | Instruction to continue past the briefed endpoint to create spacing before the next act or landing. Usage: Used in recovery sequences before CLEARED TO LAND. |
| --- | --- |

Separation and Sequencing Directives

| HOLD Separation and Sequencing Directives | Instruction to stop the current maneuver or approach and remain clear of the stated area. Requires mandatory readback by callsign. Usage: 'Mustang, HOLD, west of show center, 1,500.' |
| --- | --- |

| CLEARED IN Separation and Sequencing Directives | Authorization for a performer to enter the aerobatic box and begin the act. Always stated with direction. Usage: 'Cobra, CLEARED IN, left-to-right.' |
| --- | --- |

| RECOVER Separation and Sequencing Directives | Instruction to terminate the current act, climb to a safe altitude, and exit the box. Usage: Issued before CLEARED TO LAND. |
| --- | --- |

| SEQUENCE Separation and Sequencing Directives | A call that establishes the order of multiple performers entering the box. Usage: Use when managing two or more acts in close succession. |
| --- | --- |

Traffic and Conflict Terms

| TRAFFIC Traffic and Conflict Terms | Advisory call identifying a conflicting aircraft by callsign and position. Requires mandatory readback: VISUAL or PADLOCKED. Usage: 'Cobra, TRAFFIC, west, 1,500, inbound.' |
| --- | --- |

| CONFLICT Traffic and Conflict Terms | Stronger than TRAFFIC; used when separation is immediately threatened. Requires mandatory readback and immediate action. Usage: Use when TRAFFIC alone is insufficient to prevent conflict. |
| --- | --- |

| VISUAL Traffic and Conflict Terms | Performer acknowledgment that the called traffic is in sight and the performer is maintaining visual separation. Usage: Required response to TRAFFIC or CONFLICT. |
| --- | --- |

| PADLOCKED Traffic and Conflict Terms | Performer acknowledgment that the called traffic is in sight and the performer is dedicated to tracking it. Usage: Use when performer is actively maneuvering to maintain visual. |
| --- | --- |

Energy and Maneuver Terms

| ALTITUDE RESTRICTION Energy and Maneuver Terms | A specific altitude floor or ceiling stated as a limitation for the current or next maneuver. Usage: Example: 'Cobra, altitude restriction, 1,500 minimum.' |
| --- | --- |

| HARD STOP Energy and Maneuver Terms | A directive to immediately cease all maneuvers and hold altitude. More urgent than HOLD. Usage: Reserve for imminent safety-of-flight situations short of KNOCK IT OFF. |
| --- | --- |

Administrative and Status Terms

| STANDBY Administrative and Status Terms | Instruction to wait for a clearance; transmission to follow. Not a substitute for HOLD. Usage: Use only when a brief pause is needed before a clearance. |
| --- | --- |

| ROGER Administrative and Status Terms | Acknowledgment that a transmission was received. Does not confirm compliance — not a readback. Usage: Do not accept ROGER as a readback for HOLD or TRAFFIC. |
| --- | --- |

| WILCO Administrative and Status Terms | Acknowledgment that a transmission was received and the instruction will be complied with. Usage: Acceptable for administrative calls; not for emergency directives. |
| --- | --- |

| SHOW TIME Administrative and Status Terms | Announcement that the aerobatic box is open and the first act is cleared to begin on sequence. Usage: Transmitted on the show frequency at the start of the CoW/A window. |
| --- | --- |

Ground, Launch, and Recovery Terms

| LINE UP AND WAIT Ground, Launch, and Recovery Terms | Instruction to taxi onto the runway and hold for takeoff clearance. Air boss callsign prefix required. Usage: 'Boss, [callsign], line up and wait, Runway [XX].' |
| --- | --- |

| CLEARED FOR TAKEOFF Ground, Launch, and Recovery Terms | Authorization to take the runway and depart. Requires full readback of callsign and runway. Usage: 'Boss, [callsign], cleared for takeoff, Runway [XX].' |
| --- | --- |

| CLEARED TO LAND Ground, Launch, and Recovery Terms | Authorization to land on the designated runway. Requires readback. Usage: 'Boss, [callsign], cleared to land, Runway [XX].' |
| --- | --- |

| KNOCK IT OFF Ground, Launch, and Recovery Terms | The single emergency-stop directive: cease the maneuver, climb to a safe altitude, and acknowledge. Spoken three times. Every affected aircraft acknowledges by callsign. The air boss must confirm all callsigns before clearing the airspace. Usage: Mandatory: spoken exactly three times. All callsigns must respond. |
| --- | --- |

Usage Principles — Quick Reference

| One term, one meaning | Never substitute informal phrasing ('the line') for a defined term (SHOW LINE or DEADLINE). |
| --- | --- |

| Crowd-perspective directions | All left/right calls are from the spectators' viewpoint, never the pilot's. |
| --- | --- |

| Mandatory readback | Required for any TRAFFIC, CONFLICT, HOLD, or KNOCK IT OFF directive, by callsign. |
| --- | --- |

| Standard word order | Callsign · directive · reference · value. Example: 'Cobra, HOLD HIGH, north of show center, 2,000.' |
| --- | --- |

| Air boss callsign prefix | All ground, launch, and recovery calls are prefixed with the air boss callsign to keep the controlling authority unambiguous. |
| --- | --- |

Additional Notes — Page 1

Additional Notes — Page 2

Additional Notes — Page 3